Here is my code without a controller loop.  It sets the servo based on the slide switches with max and min limits imposed and it runs the Ping Sensor and sends 16 bit Hex result to the Right 4, 7-Seg indicators.  I will update the code over the next week. Top module and 3 sub-modules for now.

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company: UTSA

// Engineer: Morton

//

// Create Date: 11/08/2024 02:33:37 PM

//////////////////////////////////////////////////////////////////////////////////

module BBa(clk100,sw,seg,disp,PWM,Trig,Echo,red1,TestA1);

input clk100;

input [15:0] sw;

output  [7:0] seg;

output  [7:0] disp;

output PWM;

output Trig;

input Echo;

output red1;

output TestA1;

reg [31:0]count;

reg [3:0] NextState;

wire [3:0] State;

reg RTrig;

reg [31:0] RPeriod;

wire [31:0] Period;

assign State = NextState;

assign Trig = RTrig;

initial begin

NextState <= 4'h0;

end

assign TestA1 = count[19];

PWM_Gen IN105 (clk100,{sw,4'b0000}, PWM);  //Replace the concatination with the PID output

always @(posedge clk100) begin //This holds the state machine for the project tick "posedge count[19]"

count <= count+1;

case (State)  // n states

4'h0:  begin  //Initilize Trig, Interval others

RTrig = 1'b0;   //Use blocking here

RPeriod = 1'b0;

count <= 32'h00000000;  //Reset count

NextState = 4'h1;  //Jump to the next state

end

4'h1:  begin

if (count < 32'd1_000) begin

RTrig <= 1'b1;

RPeriod <= 32'h00000000;

end

else if ((count >= 32'd1_000) && (count < 32'd6_000_000)) begin

RTrig <= 1'b0;

if(Echo == 1) RPeriod <= RPeriod +32'd1;

end

else begin

NextState <= 4'h2;

end

end

4'h2:  begin

NextState <= 4'h0;

end

endcase

end

SSEG IN106 (seg,disp,clk100,RPeriod);

endmodule

// ***********************************************************

module PWM_Gen(clk100, Input, PWM_Pulse);

input clk100;

input [63:0] Input;

output reg PWM_Pulse;

reg [63:0] Count;

//    reg [63:0] RInput;

wire [63:0] RInput;

initial begin

Count = 0;

PWM_Pulse = 0;

end

always@(posedge clk100) begin

Count <= Count + 64'd1;

if(Count == 64'd600_000) Count <= 64'd0000000;  // ***Changed to 6ms period

end

//Limits PWM output to range of 1ms to 2ms = useful range

assign RInput = (Input < 64'd100_000)? 64'd100_000:((Input > 64'd200_000)? 64'd200_000:Input);

always@(*) begin

if(RInput > Count) PWM_Pulse = 1;

else PWM_Pulse = 0;

end

endmodule

/ ******************************************************

module SSEG (seg,disp,clk100,BCD);

input wire clk100;

input wire [15:0] BCD;

output reg [7:0]seg;

output reg [7:0] disp;

reg [3:0] Location = 0;

reg [3:0] Digit;

initial begin

seg = 8'b00000000;

disp = 8'b00000000;

Location = 0;

Digit = 0;

end

wire Clk_Multi;

CLK100MHZ_divider IN432476(clk100, Clk_Multi);

always@(*) begin

if(Digit == 4'b0000) seg[6:0] = 7'b1000000; // Make 0 with LEDs

if(Digit == 4'b0001) seg[6:0] = 7'b1001111; // Make 1 with LEDs

if(Digit == 4'b0010) seg[6:0] = 7'b0100100; // Make 2 with LEDs

if(Digit == 4'b0011) seg[6:0] = 7'b0110000; // Make 3 with LEDs

if(Digit == 4'b0100) seg[6:0] = 7'b0011001; // Make 4 with LEDs

if(Digit == 4'b0101) seg[6:0] = 7'b0010010; // Make 5 with LEDs

if(Digit == 4'b0110) seg[6:0] = 7'b0000010; // Make 6 with LEDs

if(Digit == 4'b0111) seg[6:0] = 7'b1111000; // Make 7 with LEDs

if(Digit == 4'b1000) seg[6:0] = 7'b0000000; // Make 8 with LEDs

if(Digit == 4'b1001) seg[6:0] = 7'b0011000; // Make 9 with LEDs

if(Digit == 4'b1010) seg[6:0] = 7'b0001000; // Make A with LEDs

if(Digit == 4'b1011) seg[6:0] = 7'b0000011; // Make b with LEDs

if(Digit == 4'b1100) seg[6:0] = 7'b1000110; // Make C with LEDs

if(Digit == 4'b1101) seg[6:0] = 7'b0100001; // Make d with LEDs

if(Digit == 4'b1110) seg[6:0] = 7'b0000110; // Make E with LEDs

if(Digit == 4'b1111) seg[6:0] = 7'b0001110; // Make F with LEDs

end

always@(posedge Clk_Multi) begin

Location <= Location + 1;

case(Location)

0:  begin

disp <= 8'b11111110;

{seg[7],Digit} <= {1'b1,BCD[3:0]};

end

1:  begin

disp <= 8'b11111101;

{seg[7],Digit} <= {1'b1,BCD[7:4]};

end

2:  begin

disp <= 8'b11111011;

{seg[7],Digit} <= {1'b1,BCD[11:8]};

end

3:  begin

disp <= 8'b11110111;

{seg[7],Digit} <= {1'b1,BCD[15:12]};

end

4:  begin

disp <= 8'b11111111;

end

5:  begin

disp <= 8'b11111111;

end

6:  begin

disp <= 8'b11111111;

end

7:  begin

disp <= 8'b11111111;

Location <= 0;

end

endcase

end

endmodule

// ************************************************************************

module CLK100MHZ_divider(clk100, New_Clock);//New_Clock = 100M/10,000 = 10KHz

input wire clk100;      // Input clock signal

output reg New_Clock;         // Divided clock output

reg [31:0] count = 32'b00000000; // [31:0] is large enough to hold any value

initial begin

New_Clock = 0;

count = 0;

end

always @(posedge clk100) begin

count <= count + 1;   // Increment count

if (count == 31'd10000) begin

New_Clock <= ~New_Clock; // Toggle New Clock

count <= 31'b0;       // Reset count

end

end

endmodule

// ***********************************************************